EfkaApp Bluetooth

# Bluetooth Modul

## Model

WINGONEER HC-06 Wireless-4 Pins Bluetooth HF-Transceiver Serial Module 4 Satz-Kabel für Arduino (7,99€ Amazon)

Product:
Standard-Baudrate: 9600,8,1, n.
Eingebaute Antenne.
Reichweite von bis zu 30 Fuß.
Bluetooth Version: V2.0 + EDR
Betriebsspannung: 3.3 V
Standard-Baudrate: 9600,8,1, n.
Signalbereich: 30ft
Artikelgröße: 4.3 * 1.6 * 0.7cm
Artikelgewicht: 3g
Packungsgröße: 9 * 3 * 1cm
Paketgewicht: 8g
Commercial Serie: Bluetooth-Modul Bord-Serie
Mit LED-Anzeige Licht, verwenden 150mA und 3.3V Regelung Chip.
Mit VCC.GND.TXD.RXD Fuß für die Bluetooth
Mit "Re-Suchanfrage" Taste (ON / OFF / WAKE Fuß für sie, externen MCU Ausgang "High Level" kann Modul steuern erneut Suchanfrage)
Kompatibel mit Bluetooth-Master-Modul ".slave Modul" oder Master-Slave (ganz) Modul.
Eingangsspannung: 3.3 ~ 6V
Größe: 1.55cm * 3.98cm
Hinweis:

Package include:
1xWINGONEER HC-06 Bluetooth-Modul
1x dupont Kabel

## Connection

RXD (white) Pin2

TXD (brown) Pin3

RXD (black) Pin5

RXD (red) Pin4

You have to connect the male SUB-D 9pin plug to the B18 plug of our control box:

# EfkaApp

The manual and a description how to install you will find in the download area of our homepage:

https://www.efka.net/index.php?id=344&L=0

| After Installation you have to check that Bluetooth is activated on your smartphone | When you scan the area you will find the module you have to “pair” |
| --- | --- |
| Enter code “1234” to pair the device | Now the device is paired |
| Start the EfkaApp and press “settings” | Press “Reload Bluetooth devices” |
| Select the Bluetooth device | Press settings button to return to main display |
| Press „connect“ | The mobile has to be connected to internet! Then press „start“ |
| Wait until connection is established | When connection to the server is established you will get a session code. This session code has to be entered on the Efka Remote homepage |

# Efka Remote

When you are connected to the control box by smartphone you can enter the connection code on our homepage:

http://remote.efka.net

After press “connect” you have full access to the control box.